Project ID: 31

A

Stage-II

Project Report

On

“Design and Fabrication of shaft driven bicycle”

By

Anish Yuvraj Patil

Harshad T Thavil

Guided by

Prof. Amit Bhagure

K.K. Wagh Education Society’s

K.K. Wagh Institute of Engineering Education and Research, Nashik-3

Department of Mechanical Engineering

AcademicYear: 2022-23

Sem-VII

# C E R T I F I C A T E

This is to certify that the project report entitled

“DESIGN AND FABRICATON OF SHAFT DRIVEN BICYCLE”

Submitted by

Anish Patil B190130935

Harshad Thavil B190131020

is a bonafide work carried out by them under the supervision of Prof. Amit Bhagure

and it is approved for the partial fulfillment of the requirement of Savitribai Phule Pune University for the award of the Degree of Bachelor of Engineering (Name of Programme)

This project report has not been earlier submitted to any other Institute or University for the award of any degree or diploma.

Internal Guide 							Head of Department

External Examiner

Date:

Place: Nashik

AKNOWLEDGEMENT

Acknowledgement is a sweet and short way to express gratitude. I take these opportunity heartfelt thanks to all those who have guided, supported and encouraged me to complete my research work.

Indeed the words at my command are not adequate to convey the depth of my feeling and gratitude to my project guide Prof. Amit Bhagure for his most valuable and inspiring guidance with his friendly nature, love and affection, for his attention and magnanimous attitude right from the first day, constant encouragement, enormous help and constructive criticism throughout the course of this investigation and preparation of this manuscript.

I am also thankful to (Project Coordinator), for counsel generous guidance and useful suggestions; special thanks are tendered to , Head of Mechanical Department.

Taken deep appreciation is being rendered to Dr. K. N. Nandurkar, Principal, K.K.Wagh Institute of Engineering Education and Research, Nashik, for providing the facilities during the course of my studies.

I would like to thank the entire staff members of Mechanical Department for timely help and inspiration for completion of the dissertation.

My vocabulary fail to get words expressed for my respect and sense of gratitude to my beloved parents, colleagues and friends who always wanted my success, inspired me with their love and affections and for the sacrifice made by them to shape my career.

|  | Mr. Anish Patil (A42) Mr. Harshad Thavil (A69) |
| --- | --- |

Acknowledgement	I

Abstract	II

List of figures	III

| Chapter No | Chapter Name | Page no |
| --- | --- | --- |
| 1 | Introduction | 01 -04 |
|  | 1.1 General introduction | 01 |
|  | 1.2 Purpose of the drive shaft | 02 |
|  | 1.3 Functions of the drive shaft | 02 |
|  | 1.4Objectives of dynamic bicycle | 04 |
| 2 | Literature Review | 05-09 |
|  | 2.1 Introduction | 05 |
|  | 2.2 History | 05 |
|  | 2.3 Different types of shafts | 06 |
|  | 2.4 Demerits of conventional driveshaft | 08 |
|  | 2.5 Merits of composite driveshaft | 08 |
|  | 2.6 Drive shafts vibration | 08 |
| 3 | Drive Mechanism | 10-16 |
|  | 3.1 Introduction | 10 |
|  | 3.2 Types of gears | 11 |
|  | 3.3 List of gears | 11 |
|  | 3.4 Backlash | 14 |
|  | 3.5 Shifting of gears | 15 |
|  | 3.6 Tooth profile | 15 |
|  | 3.7 Gear materials | 16 |
|  | 3.8 The module system | 16 |
| 4 | Design of cast iron drive shaft | 17-25 |
|  | 4.1 Design assumptions | 21 |
|  | 4.2 Design Optimization | 25 |
| 5 | Manufacturing Process | 26-29 |
|  | 5.1 Introduction | 26 |
|  | 5.2 Casting Process | 26 |
|  | 5.3 Forming process | 26 |
|  | 5.4 Fabrication Process | 26 |
|  | 5.5 Material Removal Process | 28 |
|  | 5.6 Welding | 29 |

## Advantages and Limitations	30

## Troubleshooting.	31

## Conclusion	32

## References

ABSTRACT

This project is developed for the users to rotate the back wheel of a two wheeler using propeller shaft.

Power transmission through chain  drive  is  the  oldest  and  widest  used  method  in  case of bicycle. In this paper we implemented the chainless transmission to the bicycle  to overcome the various disadvantages of chain drive. Shaft drives were introduced over a century ago, but were mostly supplanted by chain-driven bicycles due to the gear ranges possible with sprockets and derailleur. Recently, due to advancements in internal gear technology, a small number of modern shaft-driven bicycles have been introduced. Usually in two wheelers, chain and sprocket method is used to drive the back wheel.

The shaft drive only needs periodic lubrication using a grease gun to keep the gears running quiet and smooth.

A shaft driven bicycle is a bicycle that uses  a  shaft  drive  instead  of  a  chain  which  contain two set of bevel gear at both the ends to make  a  new  kind  of  transmission system for bicycle for getting high reliability system, and more safesystem.Shaft-driven bikes have a large bevel gear where  aconventional  bike  would  have  its chain ring. This meshes with another bevel gear mounted on the driveshaft.The use of bevel gears allows the axis of the drive torque from the pedals to be turned through 90degrees.The drive shaft then  has  another  bevel  gear  near  the  rear  wheel  hub  which  meshes with a bevel gear on the hub where  the  rear sprocket would  be on a conventional bike, and cancelling out the first drive torque change ofaxis.

According tothe direction of motion of pedal, the wheel will be moved forwardorreverse. This avoids the usage of chain and sprocket method.This “chainless” drive system  providessmooth,  quiteand  efficienttransferofenergy from the pedals to the rearwheel.It is attractive in look compare with chain driven bicycle.  It  replacesthe  traditional  method.

Keyword: - Shaft, Bevel gear

| Figure No | List of figure | Page no |
| --- | --- | --- |
| 1.1 | Replacement of chain drive bicycle with driveshaft | 01 |
| 1.2 | 3D model ofadriveshaft | 03 |
| 1.3 | Positionofdriveshaft | 04 |
| 1.4 | Parts ofdriveshaft | 07 |
| 1.5 | BevelGear | 13 |
| 4.1 | Assembling ofbearingcover | 17 |
| 4.2 | Assembling ofbevelgears | 19 |
| 4.3 | Assembly of pedalwith gears | 20 |
| 4.4 | Bevelgearmechanism | 20 |
| 5.1 | Drilling Operations | 28 |
| 5.2 | Turning Operations | 28 |
| 5.3 | Grinding Operation | 28 |
| 5.4 | Milling Operation | 28 |

List of Figure

# LIST OF TABLES

## Tables	Page No. Chapter-4

  - Design  requirements andspecification	20

  - Mechanical  properties  of castiron(SM45C)	21

CHAPTER- 1

INTRODUCTION

## GENERAL INTRODUCTION

- A shaft-driven bicycle is a bicycle that uses  adrive  shaft  instead  of  a  chain  to  transmit power from the pedals to the rear wheel.  Shaft  driveswere  introduced  over  a century ago, but were mostly supplanted by chain-driven bicycles due to the gear ranges possible with sprockets and derailleur.  If  bevel-gear  could  be  accurately  and  cheaply  cut  by  machinery, it is possible that gears of this description might supplant, to a greatextent.

Shaft-driven bikes have a large bevel gear  wherea  conventional  bike  would  have  its chain ring. This meshes with another bevel gear  mountedon  the  drive  shaft.  The  useof  bevel gears allows the axis of the drive torque from the pedals to  be  turned  through  90 degrees.The drive shaft thenhas  anotherbevel  gear  near  the  rear  wheel  hub  which meshes with a bevel gear on the hub where the rear  sprocket  would  be on a conventional  bike, and cancelling out the first drive torque change ofaxis.

Figure 1.1- Replacement of chain drive bicycle with driveshaft

The design of bevelgear produceslessvibrationandlessnoisethan conventional straight-cut or spur-cut gear with the straight teeth. The  shaft  drives  only  needs periodic lubrications using a grease gun to  keep  the  gears  running  quite,  smooth and efficient transfer of  energy  from  the  pedals  to  the  rear  wheel.  It  isattractive  in look compared with chain drivenbicycle.

An automotive drive shaft  transmits  power  from  the  engine  to  the  differential gear of a  rear  wheel  drive  vehicle.  The  drive  shaft  is  usually  manufactured  in  two pieces toincrease the fundamental bendingnaturalfrequency  because the  bending natural frequency of a shaft is inversely proportional to the square of beam length and proportional to the square root  of specific  modulus  which  increases  the  total weightof an automotive vehicle and decreases fuel efficiency. So, a single piecedrive  shaft  is preferred here and the material of it is  considered  to  be  Titanium  alloy  because  of  its high strength and low density. Drive shafts are carriers  of  torque  and  are  subject  to torsion and shear stress, equivalent to the difference between  the  input  torque  and  the load. They must therefore be strongenough  tobear  the  stress, whilst  avoiding  too much additional weight as that would in turn increase theirinertia.

  - Purpose of the Drive Shaft (Or PropellerShaft)

The torque that is produced fromthe  engine  and  transmission  must  be transferred to the rear wheels to push the vehicle forward  and  reverse.  The  drive  shaft must provide a smooth, uninterrupted flow of power to the axles. The drive shaft and differential are used to transfer thistorque.

## Functions of the DriveShaft

    - First, it must transmit torque from the transmission to the differential gearbox.

    - During the operation,it  isnecessary  to  transmit  maximum  low-gear  torque  developed by theengine.

    - The drive shafts must also be capableof  rotatingat  the  very  fast  speeds  required  by thevehicle.

    - The drive shaft must also operate through constantly changing angles between the transmission, the differential and the axles. As the  rearwheels  roll  over  bumps  in  the road, the differential and axles move up and down. This movement changes the angle between the transmission and thedifferential.

    - The length of the  drive  shaft  must  also  be  capable of  changingwhiletransmitting torque. Length changes are caused by axle movement due to torque reaction, road deflections, braking loads and so on. A slip joint is used to compensate forthis

Figure 1.2- 3D model of a drive shaft

motion. The slip jointisusuallymadeof  aninternal  and  external  spline.  It is  locatedon the front end of the drive shaft and is connected to thetransmission.

Now days  all  automobiles  (which  arehaving  front  engine  rear  wheel  drive) have the transmission  shaft  as  shown  in  figure.  A  pair  of  short  drive  shafts  is commonly used to send power from  a  central  differential,  transmission,  or  transaxle  to the wheels. Two piece drive shaft increases the weight  ofdrive  shaft  which  is  not desirable in today’s market. Many methods are available at present for the design optimization of structural systems and these methodsbased on  mathematicalprogramming techniques involving gradient search and direct search. The reductionin weight of the drive system is advantageous in overall weight  reductionof  automobiles which is a highly desirable goal of designengineer.

Figure   1.3- Position of Drive Shaft

## Objectives of dynamicbicycle

    - Increase durability of bicycle with the help of shaftdrive.

    - Reduce maintenance cost ofbicycle.

    - Increase power transmission efficiency of thebicycle.

    - Making system morereliable.

    - Reduce noise pollution using shaft drive transmissionsystem.

CHAPTER- 2

LITERATURE REVIEW

## INTRODUCTION

    - Drive shafts are carriers of torque; they are subject to torsion and shear stress, which represents the difference between the input force and the load.  They  thus  need  to  be strong enough tobear the stress, without imposing  too  great  an  additional  inertia  by virtue of the  weight  of  the  shaft.  Most  automobiles  today  use  rigid  driveshaft  to deliver power from a transmission  to the  wheels.  A  pair  of  short  driveshaft  is commonly used to send power from  a  central  differential,  transmission,  or  transaxle  to the wheels. There are different types of drive shafts in AutomotiveIndustry:

- 1 piecedriveshaft

- 2 piecedriveshaft

- Slip in Tubedriveshaft

The Slip in Tube Driveshaft is the new type which also helps in Crash Energy Management. It can be compressed in case of crash. It is  alsoknown  as a collapsible drive shaft. Front-wheel drive is the most  commonform  of  engine/transmission  layout used in modern passenger cars, where the engine drivesthefrontwheels.Mostfront wheel drive vehicles  today  feature  transverse  engine  mounting,  where  as  in  past decades  engines  were  mostly  positioned  longitudinally  instead.  Rear-wheel  drivewas the traditional standard and is still widely used in luxury cars and most sportcars.

## History

    - The first shaft drives for  cyclesappear  to  have  been invented  independently in 1890 in  the United States and England. A. Fearnhead, of 354 Caledonian Road, North London developed one in 1890 and received a patent in October 1891.His prototype shaft was enclosed within a tube running along the top  of  the  chain  stay;  later  models  were enclosed within the actual chain stay .In the United States, Walter Still man  filed  for  a patent on a shaft-driven bicycle on Dec. 10, 1890 which was granted on July 21,1891.

    - The shaft drive was not well accepted in England, so in 1894 Fearnhead took it to  theUSA where Colonel Pope of the Columbia firm bought the exclusive American rights. Belatedly,  the  English  makers  took  it  up,  with   Humber in  particular  plunging   heavily  on the deal. Curiously enough,  the  greatest  of  all  the  Victorian  cycle  engineers, Professor Archibald Sharp, was against shaft drive; in his classic 1896 book"Bicycles

and Tricycles", he writes "The Fearnhead Gear....  if  bevel-wheels  could  be  accurately and cheaply cut by  machinery,  it  is  possible  thatgears  of  thisdescription  might supplant, to a great extent, the chain-drive gear; but the fact that the teeth of the bevel- wheels cannot be accurately milled is a serious obstacle to their practicalsuccess".

    - In the USA, they had been made by the  League  Cycle  Company  as  early  as  1893. Soon after,the  Frenchcompany  Metro  pole  marketed  their  Acatane.By1897 Columbia began aggressively tomarket the chainless bicycle it had acquired fromthe League Cycle  Company. Chainless bicycles  were  moderately  popular  in  1898  and  1899,  although  sales  were  still  much  smaller  than  regular   bicycles,   primarily  due  to the high cost. The bikes were  also  somewhat  less  efficient  than  regular  bicycles:  there wasroughly an 8 percentloss in the gearing, in part due to limited manufacturingtechnology at the time.  The  rear  wheel  was  also  more  difficult  to  remove  to  change flats. Many of these deficiencies have been overcome in the pastcentury.

    - In 1902, The Hill-Climber Bicycle  Mfg.  Company  solda  three-speed  shaft-drivenbicycle in which the shifting was implemented with  three  sets  of  bevel  gears.  While  asmall number of chainless bicycles  were  available,  forthe  most  part,  shaft-driven  bicycles disappeared from view for most of the 20th century. There  is,  however,  still a niche market for chainless bikes, especially for commuters, and there are a number of manufacturers who offer them either as part of a larger range or  as  a  primary specialization. A notable example is Bio mega inDenmark.

## Different Types ofShafts

- Transmissionshaft: These shaftstransmitpowerbetween thesourceandthe machines absorbing power. The  countershafts,  line  shafts,  overhead  shafts  and  all factory shafts are transmission shafts. Since these shafts carry machinepartssuchas pulleys, gears etc., therefore they are subjected to  bendingmoments  in  addition  to twisting.

- Machine Shaft: These shafts form an  integralpart  of the  machine  itself.  For example, the crankshaft is an integral part of I.C engines slider-crankmechanism.

- Axle: A shaft is caled“an axle”,  if  it  is  a  stationary machine  element  and  is  used  for the transmission of bending moment only.  It  simplyacts  as  a  support  for  rotating  bodies.

Application: To support hoisting drum, a car wheel or a rope sheave.

- Spindle:  A  shaft  is  called  “a  spindle”,  if  it  is  a  short  shaft  that  imparts  motion either to a cutting tool or to awork-piece.

Applications:

  - Drill press spindles-impart motion to cutting tool (i.e.)drill.

  - Lathe spindles- impart motion towork-piece.

Apartfrom, an axle and a spindle, shaftsareusedatsomanyplacesandalmost everywhere wherever power transmission is required. Few of themare:

- Automobile Drive  Shaft: Transmits power from  main  gearbox  to  differential gearbox.

- Ship Propeller Shaft: Transmits power  fromgearbox topropellerattachedonit.

- Helicopter Tail Rotor Shaft: Transmits power to rail rotorfan.

Part of Drive Shaft

Figure   2.1- Parts of Drive Shaft

  - Demerits of a Conventional DriveShaft

- They have less specific modulus andstrength.

- Increasedweight.

- Conventional steel drive shafts are usually manufactured in two pieces to increase the fundamental bending natural  frequency  because the  bending  natural  frequency  of  a shaft is inversely proportional to the square of  beam  length  and  proportional  to  the square root of specific  modulus.  Ther1efore  the  steel  drive  shaft  is  made  in  two sections connected by a support structure, bearings andU-joints  and  hence  over  all weight of assembly will bemore.

- Its corrosion resistance is less as compared with compositematerials.

- Steel drive shafts have less dampingcapacity.

## Merits of Composite DriveShaft

- They have high specific modulus andstrength.

- Reducedweight.

- The fundamental natural frequency of the carbon  fiber  composite  drive  shaft  can  be twice as high as that of  steel  or  alluminiumbecause the  carbon  fiber  composite material has more  than4  times  the  specific  stiffness  of  steel  or  alluminium,  which makes it possible to manufacture the drive shaft of passenger cars in one piece.  A one- piece composite shaft can be manufactured so as  tosatisfy  the  vibration  requirements. This eliminates all the  assembly,  connecting  the  two  piece  steel  shafts  and  thus minimizes the overall weight, vibrations and the totalcost

- Due to the weight reduction, fuel consumption will bereduced.

- They have high damping capacity hence they produce less vibration andnoise.

- They have good corrosionresistance.

- Greater torque capacity than steel or alluminiumshaft.

- Longer fatigue life than steel or alluminiumshaft.

- Lower rotating weight transmits more of availablepower.

  - Drive ShaftVibration

    - Vibration is the most  commondrive  shaft  problem.  Small  cars  and  short  vans  and trucks (LMV) are able to  use a single drive  shaft  with  a  slipjointatthefrontend without  experiencing  any  undue  vibration.  However,  with  vehicles  of  longer  wheel base, the longer drive shaft required would tend to sag and under certain operatingconditions would tend to whirl and then setup resonant vibrations in  the  body  of  the vehicle, which will cause the body to vibrate as the shaftwhirls.

    - Vibration can be either transverse or torsional. Transverse vibration is the result of unbalanced condition acting on the shaft. This condition is usually  bydirt  or  foreign material on the shaft, and it can cause a rather  noticeable  vibration  in  the  vehicle.  Torsional vibration occurs from the power impulses of the engine or  from  improper universal join angles. It causes a noticeable  sounddisturbance  and  can  cause  a mechanical shaking. In excess, both types of  vibrationcan  cause  damage  to the universal joints and bearings. Whirling of a rotating shaft happens when  the  centreof gravity of the shaft mass is eccentric and so is acted  upon by a  centrifugal force  which tends to bend or bow the shaft so that it orbits about the shaft longitudinal  axis  like  a rotating skipping rope. As  the  speed  rises,  the  eccentric  deflection  of  the  shaft increases, with the result that the  centrifugal  force  also  will  increase.  The  effectis thereforecumulative  and  will  continue  until  the  whirling   become   critical,   at   which point the shaft will vibrateviolently.

    - From the theory of whirling, it has  been  found  that  the  critical  whirling  speed  of  the shaft is inversely proportional to the square of  the  shaft  length.  If,  therefore,  a  shaft having,  for  example,  a  critical  whirling  speed  of  6000  rev/min  is  doubled   in  length,  the critical whirling of the new shaft  will  be  reduced  to  a  quarter  of this,  i.e.  the  shaftwill now begin to  rotate  at 1500 rev/min. Thevibrationproblemcouldsolveby increasing the diameter of  the  shaft,  but  this  would  increase  its  strength  beyond  its torque carrying requirements and at the  same  time  increase  its  inertia,  which  would oppose the  vehicle’s  acceleration  and  deceleration.  Another  alternative  solution frequently adopted by car, van, and commercial vehicle manufacturers is the use of two-piecedrive shafts supportedby intermediateorcentrebearings.Butthiswill increase the costconsiderably.

09

CHAPTER- 3

DRIVE MECHANISM

## INTRODUCTION

    - For the gear-like device used to drive a roller chain, see Sprocket. This article is about mechanical gears. For other uses, see Gear  (disambiguation)Two  meshinggears transmitting  rotational  motion.  Note  thatthe  smaller  gear  is  rotating  faster.   Although the  larger  gear  is  rotating  less  quickly,  its  torque  is  proportionally  greater.   One subtlety of this particulararrangementisthatthelinearspeedatthepitchdiameteris the same on bothgears.

    - A gear or cogwheel is a rotating machine part having cut teeth,or cogs,whichmesh with another toothed part in order to  transmit torque,  in  most  cases  with  teeth  on the one gear being of identical shape,  and  often  also  with  that  shape  on  the  other  gear. Twoor  more   gears   working  in   tandem  are   called   a transmission and   can  produce a mechanical advantage through a gear ratio and  thus  may  be  considered  a simple machine. Geareddevices can change thespeed,torque,anddirectionof  apower source. The most  common  situation  is  for  a  gear  to  mesh with  another  gear;  however, a gear can also mesh withanon-rotatingtoothedpart,calledarack,therebyproducing translation instead ofrotation.

    - The gears in a  transmission  are  analogous  to the  wheels  in  a  crossed  belt pulley  system. An advantage of gears is that the teeth of a gear  preventslippage.  When  two gears mesh, and one gear is  bigger  than  the  other  (even  though  the  size  of  the  teeth must match), a mechanical advantage is produced, with the rotational  speeds and  the torques of the two gears differing in an inverserelationship.

    - In transmissions which  offer  multiple  gear  ratios,  such  as  bicycles,  motorcycles,  and cars, the term gear, as in first gear, refers to a gear ratio rather than an actual physical gear.   The   term is used   to describe    similar    devices   even   when   the    gear   ratio is continuous rather than discrete, or when  the  device  does  not  actually  contain  any gears,  as  in  a continuously  variable  transmission.  The  earliestknown   reference   to  gears  was   circa  A.D.  50  by Hero  of  Alexandria, but   they  can  be  traced  back  to  the Greek mechanics of the  Alexandrian  school in  the  3rd  century  B.C.  and  were greatly   developed  by  the    Greek polymath Archimedes (287–212   B.C).The Antikythera mechanism is an example of a very early and intricate geareddevice,

10

designed to calculate astronomical positions. Its time of construction is now estimated between 150 and 100 BC.

    - The definite velocity ratio which results from having teeth gives gears anadvantage

over other drives (such as traction drives and V-belts) in precision machines such  as watches that depend upon an exact  velocity  ratio.  In cases  wheredriver  and  follower are proximal, gears also have an advantage over other drives in the reduced  number  of parts required; the downside is that gears are more expensive to manufacture and their lubrication requirements may impose a higher operating cost.

## Types ofgears

An external gear is one with the  teethformed  on the  outer  surface  of a cylinder or cone.Conversely,

An internal gear is one with the  teethformed  on  the  inner  surface  of a  cylinder or cone. For bevel gears, an internal  gearis  one  with  the pitch angle  exceeding 90 degrees. Internal gears do not cause output shaft directionreversal.

## List ofgears

1) Spur gear

Spur gears or straight-cut gears are the simplest type of gear. They consist of a cylinder or disk with the teeth  projecting  radials,  and  although  they  are  not  straight- sided  in  form  (theyare  usually  of  special  formto achieve  constant  drive   ratio, mainly involute), the edge of each tooth is straight and aligned parallel to  the  axis  of rotation. These gearscan  bemeshed  together  correctly  only  if  theyare  fitted  to parallelshafts.

    - Helicalgears

Helical or "dry fixed" gears  offera  refinement  over  spur  gears.  The  leadingedges of the teeth are not parallel  to  the  axis  of rotation,  but are  set at an angle.  Since the gear is curved, this angling  causes  the  tooth  shape  to  be  a  segment  of  a helix. Helical gears can be  meshed  unparallel or crossed orientations.  The  formerrefers  to when the shafts are parallel to each other; this is the  most  common  orientation.  In  the latter, the shafts are non-parallel,  and  in  this  configuration  the  gears  are  sometimes known as "skewgears".

The angled teeth engage more gradually than do spur gear teeth, causing them

to run more smoothly and quietly. With parallel helical  gears,  each  pair  of  teeth  first make contact at a single point at one side of the  gear wheel;  a moving  curve  of contact then grows gradually across the tooth face to a maximum then  recedes  until  the  teeth break contact at a single point  on  the  opposite  side.  In  skewgears,  teeth  suddenly meet at a line contact across their entire  width  causing  stress  and  noise.  Skew  gears make a characteristic whine at  high  speeds.  Whereas  spur  gears  are  used  for  low  speed applications and  those  situations  where  noise  control  is  not  a  problem,  the  use of helical gears is indicated when the application involves high speeds, large power transmission,  or  where noise  abatement is  important. The  speedis  considered  to   be high when the pitch line velocity exceeds 25m/s.

A disadvantage of helical gears is a resultant thrust along the  axis  of  the  gear, which needs to be  accommodated  by appropriate thrust  bearings,  and  a  greater degree of sliding friction between  the meshing teeth, often addressed with additives  in  the lubricant.

    - Skewgears

For a 'crossed' or 'skew' configuration, the gears must have  the  same  pressure angleand  normal  pitch;  however,  the  helix  angle  and   handedness  can  be  different. The relationship between the two shafts  isactually  defined  by  the  helix  angle(s)  of the two shafts and the handedness, asdefined:

Where is the helix angle for the gear? The  crossed  configuration  is  less mechanically sound because  there  is  only  a  point  contact  between  the  gears,  whereas in the parallel configuration there is a linecontact.

Quite commonly, helical gears are used with the helix angle of one havingthe negative of the helix angle  of the  other;  such  a  pair  might  also  be  referred  to  as having a right-handed helix and  a  left-handed  helix  of  equal  angles.  The  two  equal  but opposite  angles  add  to  zero:  the  angle  between  shafts  is  zero   –  that  is,   the   shafts are parallel. Where the sum or  the  difference  (as  described  in  the  equations  above)  is not zero  the  shafts  are crossed.  For  shaftscrossed at  rightangles,thehelixanglesare of the same hand because they must add to 90degrees.

    - Double helicalgears

Double helical  gears,  or herringbone  gears,  overcome  the  problem  of  axial thrust presented by "single"  helical  gears,  by having  two  sets  of teeth  that  are  set  in  a V shape. A double helical gear can be thought of as two mirrored helical gears joined together. This arrangement cancels out the net axial thrust, since each half of thegear thrusts  inthe  opposite  direction  resulting  in   a   net   axial force of zero. This   arrangement can  remove  the  need  for  thrust  bearings.  However,  doublehelical  gears are more difficult to manufacture due to their more complicatedshape.

For both  possible  rotational  directions,  there  exist  two  possible  arrangements  for the oppositely-oriented helical gears  or  gear  faces.  One  arrangementis  stable,  and the other is unstable. In a stable orientation, the helical gear  facesare  oriented  so  that each axial force is directed toward the center  of  the  gear.  In  anunstable  orientation, both axial forces are directed away from  the  center  of the  gear.  In  both arrangements, the total (or net) axial  force  on each gear  is  zero  when  the  gears  are  aligned  correctly. If the gears become misaligned in the axial  direction,  the  unstable  arrangement  will  generate a net force that may lead to disassembly of the gear train, while the stable arrangement generates a  net  correctiveforce.  If  thedirection  of  rotation  is  reversed, the direction of the axial thrusts is also reversed, so a stable  configuration  becomes unstable, and viceversa.

Stable  doublehelical  gears  can  be  directly  interchanged  with  spur  gears without any need for differentbearings.

    - Bevelgear

A bevel gear is shaped like a right circular cone with  mostof  its  tip  cut  off.  When two bevel  gears  mesh,  their  imaginary  vertices  must  occupy  the  same  point. Their shaft axes also intersect at thispoint,forminganarbitrarynon-straight  anglebetween the shafts. The angle between the shafts can be anything except zero or 180 degrees. Bevel gears with equal numbers of teeth  andshaft  axes  at  90  degrees  are called mitergears.

Figure   3.1- Bevel gears

    - Spiral bevelgear

Spiral bevel gears can be manufactured as Gleason types (circular arc with non-constant tooth depth), Oerlikon and Curvextypes (circular arc with constant tooth depth), KlingelnbergCyclo-Palloid(Epicycloids with  constanttooth  depth)  or KlingelnbergPalloid. Spiral bevel gears have the same advantages and  disadvantagesrelative to their straight-cut cousins as  helical  gears  do  to  spur  gears.  Straight  bevelgears are generally used only at speeds below 5 m/s (1000 ft/min), or,  for  small gears, 1000 rpm.

Note: The cylindrical gear tooth profile corresponds to an involute,  butthe  bevel gear tooth profile to an octoid. All traditional bevel   gear  generators(like  Gleason, Klingelnberg,  Heidenreich&Harbeck,and  WMWModule) manufacture   bevel gears with an octoidaltooth profile. IMPORTANT: For 5-axis milled bevel gear sets it is important to choose the same calculation / layout like the  conventionalmanufacturing method.Simplified  calculated  bevel  gears  on  the  basis  of  an  equivalent   cylindrical gear in normal section with an involute tooth form show  a  deviant  tooth  form  with reduced tooth strength by 10-28% without offset and  45%  with  offset [Diss.  Hünecke, TU Dresden]. Furthermore those "involute bevel gear sets" causes morenoise.

    - Hypoidgear

Hypoid gears resemble spiral bevel   gears  exceptthe  shaft  axes  do  not intersect. The pitch surfaces  appear  conical but,  to  compensate  for the  offset  shaft,  are in  fact hyperboloids of  revolution.  Hypoid  gearsare  almost   always   designed   to operate with shafts at 90 degrees.Dependingonwhichsidetheshaftisoffsetto, relative to the angling of the teeth, contact between hypoid gear teeth may  be  even smoother and more gradual than with spiral bevel gear teeth,  but  also  have  a  sliding action along the meshing teeth as it rotates  and  therefore  usually  require  some  of  the most viscous types of gear oil to  avoid  it  being  extruded  from  the  mating  tooth  faces, the oil is normally designated HP (for hypoid) followed by a  number  denoting  the  viscosity.

Also, the pinion can be designed with  fewer  teeth  than  a  spiral  bevel  pinion, with the result that gear ratios of60:1  and  higher  are  feasible  using  a  single  set  of hypoid gears. This style  of  gear  is  most  commonly  found  drivingmechanical differentials; which are normally straight cut bevel gears; in motor vehicleaxles.

## Backlash

    - Backlashis the error in motion that occurs when gears change direction. Itexists

ecausethere isalwayssomegapbetweenthetrailingfaceofthedrivingtoothand the leading face of the tooth behind it on the driven gear, and that  gap  must  be closed before force can be transferred in the new  direction.  The  term"backlash"  can alsobe used to refer to the size of the gap, not just the phenomenon it  causes; thus,  one  could speak of a pair of gears as having, for example, "0.1 mm of backlash." A pair of gears could be designed to have zero backlashes, but this would presuppose perfection in manufacturing,uniformthermalexpansioncharacteristics throughoutthe system, and no lubricant. Therefore, gear pairs are designed to have some backlash. It  isusually provided by reducing the  tooth  thickness  of each gear by half thedesiredgapdistance. In the case of a large gear and a small pinion, however, the backlash is  usuallytaken entirely off the gear and the pinion is  given  full  sized  teeth.  Backlash  canalso  be provided by moving the gears  further  apart.  The  backlashof  a  gear  trainequals  the sum of the backlash of each pair of gears, so in long trains backlash can become a problem. For situations in which precision is important, suchas  instrumentationand  control, backlashcanbeminimizedthroughoneofseveraltechniques.Forinstance, the gear can be  split  along  a  plane  perpendicular to theaxis,onehalf fixedto theshaft in the usual manner,  the  other  half  placed  alongside  it,  free  to  rotate  about  the  shaft, but with  springs  between  the  two  halves  providing  relative  torque  between  them,  so that one  achieves,  in  effect,  a  single  gear  with  expanding  teeth.  Another  methodinvolves taperingtheteethintheaxialdirectionandprovidingforthegeartobeslid in the axial direction to take upslack.

## Shifting ofgears

    - In some machines (automobiles) it is necessary to alter the gear ratio to suit the task, a process known as gear shifting or changing gear. There are several outcomes of  gearshifting  in  motor  vehicles.  In  the  case  of  vehicle  noise  emissions,there  are  highersound  levelsemitted  when  the  vehicleisengagedinlowergears.  The  design  life   of  the lower ratio gears is shorter, so cheaper gears may be used (i.e. spur  for  1st  and reverse) which tends to generate more noise due to smaller  overlap  ratio  and  a  lower mesh stiffness etc. than the helical gears used for the  high  ratios.  This  fact  has  been  utilized in analyzing vehicle generated sound since the late 1960s,  and  has  been incorporated into the simulation of urban roadway noise and  corresponding  design  of urban noise barriersalongroadways.

## Toothprofile

    - A profile is one side of a tooth in a cross section between the outside circle andthe

root circle. Usually a  profileis  the  curve  of intersectionof atooth surfaceanda plane or surface normal to the pitch surface, such as the transverse, normal, or axialplane.

The fillet  curve  (root  fillet)  is  the  concave  portion  of  the  tooth  profile  where  it  joins the bottom of the tooth space. The velocity  ratiois  dependent  on  the  profile  of  the teeth. Frictionand wear between  two  gears  is  also  dependent  on  the  tooth  profile. There are a great many tooth  profiles  that  will  give  a  constant  velocity  ratio,  and  in many cases, given an arbitrary tooth shape,  it  is  possible  to  develop  a tooth profile  for the  mating  gear  that  will  give  a  constant  velocity  ratio.  However,  two   constant velocity tooth profiles have been  by  far  the  most  commonly  used  in  modern  times. They are the cycloidand the involute. The  cycloidwas  more  common  until  the  late 1800s; since then the involute has largely superseded it, particularly in drive train  applications. The cycloid is in some  ways  the  more  interesting  and  flexible  shape; howeverthe  involute  has  two  advantages:  it  is   easier  to  manufacture,  and  it  permits the center to center spacing of the gears to vary over some range without ruining the constancy of the velocity ratio.  Cycloidal  gearsonly  work   properly  if  the  center spacing is exactlyright.

## Gearmaterials

    - Numerousnonferrous  alloys,  cast  irons,  powder-metallurgy  and  plastics   are  used  in  the manufacture of gears. However, steels  aremost  commonly  used  because  of  their high strength-to-weight ratio and low cost. Plastic  is  commonly  used  where  cost  or  weight is a concern. A properly designed plastic gear can replace steel in many cases because it has many desirable  properties,  including  dirt  tolerance,  low  speed  meshing,  the ability to  "skip" quite well and the ability to be made with materials not  needing additional lubrication. Manufacturers have employed plastic gears to reduce costs in consumer items including copy machines, optical storage devices,  cheapdynamos, consumer audio equipment, servo motors, andprinters.

## The modulesystem

    - As a result, the term module is usually understood to  meanthe  pitch  diameter  in  millimeters divided by the number of teeth. When the module is based upon inch measurements, it is known as the English module to avoid  confusionwith  the  metric module.  Moduleis  adirect  dimension,  whereas  diametral  pitch  is  an   inverse dimension (like "threads perinch").

CHAPTER- 4

DESIGN OF CAST IRON DRIVESHAFT

    - A shaft-drivenbicycle isa bicycle thatusesa drive shaft instead of a chain totransmit power from the pedals  to  the  wheel  through  contact  of gears  and  a  shaft  rod to smoothly and efficient. Shaft drives were introduced over a  century  ago,  but  were mostly supplanted by chain-driven bicycles due to   the  gear  ranges  possible  with sprockets  and  derailleurs.  Recently,   due   to  advancementsin  internal  gear  technology, a small number of modern shaft-driven bicycles have beenintroduced.

Figure 4.1- Assembling of bearing cover

The torque that is produced fromthe  engine  and  transmission  must  be transferred totherearwheelstopushthevehicleforwardmoment.Thedriveshaft must provide a smooth, uninterrupted flow of power to the axles. The drive shaft and differential are used to transfer thistorque.

Functions of the DriveShaftare :

      - It must transmit torque from the transmission to thepedal

      - During the operation, it is necessary to transmit maximum low-geartorque

      - The drive shafts must also  becapable  of rotating  at  the  very  fast  speeds  required by thevehicle.

      - The drive shaft must also operate through constantly changing gear velocity ratio.

      - The length of the drive shaft must also be capable  ofchanging  while  transmitting torque. Length changes are caused by axle movement due to torque reaction, road deflections, braking loads and so on. A slip joint isused  tocompensate  for  this motion.

      - The slip joint is usually made of an internal and external spline. It is located onthe

front end of the drive shaft and is connected to the transmission.

## Construction and workingprinciple of the shafts are :

    - The term Drive shaft is  used  to  refer  to  a  shaft,  which  is  used  for  the  transfer  of motion from one point to another. Whereas the shafts, which propel (push  theobject ahead)are referred to  as the  propeller  shafts.  However  the  drive  shaft  of  the automobile is  also  referred  to  as  the  propeller  shaft  because  apart  from  transmitting  the rotary motion from the front end to the rear end of  the  vehicle,  these  shafts  also  propel the vehicle forward. Theshaftistheprimaryconnectionbetweenthefrontand the rear end (engine and differential), which performs both the jobs of transmittingthe motion and propelling  thefront  end.  Thus  the  terms  Drive  Shaft  and  Propeller  Shafts are used interchangeably. In other words,  a  drive  shaft  is  a  longitudinal  power transmitting, usedin  vehiclewherethe  pedal  is  situated  at  the  human  feet.  A  drive shaft is an assembly of one or more tubular shafts  connected  by  universal,  constant velocity or flexible joints. The number of tubular  piecesand  joints  depends  on  the distance between the twowheels.

    - The job involved isthe  design  for  suitable  propeller  shaft  and  replacement  of  chain drive smoothly to transmit power from  the  enginetothewheelwithoutslip.Itneeds only a less maintenance. It is cost effective. Propeller shaft strength is more and  alsopropeller  shaft  diameter  is  less.  it  absorbsthe  shock.  Because  the  propeller  shaft center is fitted with the  universal  joint  is  a  flexible  joint.  It  turnsinto  any  angular position. The both end of the shaft are fitted with the bevel  pinion,  the  bevel  pinion engaged with the crown  and power is  transmitted  to   the  rear  wheel  through  the propeller shaft and gear box.. With our shaft drive  bikes;  there  is  no  more  grease  on your hands or your clothes; and no more chain and derailleurmaintenance.

    - Shaft-drivenbikes  have  a  large bevel gear where   a   conventional   bike   would   have its chain  ring.This  meshes  with  another  bevel gearmounted  on  the  drive  shaft.   The use of bevel gears allows the axis of the drive torquefromthepedalstobeturned through 90 degrees. The drive shaft  then  has  another  bevel  gear  near  the  rear  wheel hub which meshes with a bevel gear on the hub where the rear sprocket would be on a conventional bike, and canceling out the first drive torque change ofaxis.

Figure 4.3- Assembling of pedal with gear

    - The 90-degree change of the drive plane that occurs at the  bottom bracket and again  at  the rear hub uses bevel  gears for the  most  efficient  performance,though  other  mechanisms could be used,  e.g. Hobson’s  joints, worm  gears or crossed  helical  gears. The drive shaft is often mated to a hub gear which is  aninternal  gear  system  housed  inside the rearhub.

Figure 4.4- Bevel Gear Mechanism

## Specification of driveshaft :

    - The  specifications  of  the  composite  drive  shaft  of  an  automotive  transmission  are  same as that of the steel drive shaft for optimaldesign.

    - The fundamental natural  bending  frequency  for  passenger  cars,  small  trucks,  and  vans of the propeller shaft should be  higher  than  6,500  rpm to  avoid  whirling  vibration  and the torque transmission capability of the  drive  shaft  should  be  larger  than  3,500  Nm. The drive shaft outer diametershould  notexceed  100  mm  due  to  space  limitations. Here outer diameter of the shaft is taken as 90 mm.  The  drive  shaft  of  transmission  system is to be  designed  optimally  for  following  specified  design  requirements  as shown inTable

Table 4.1: Design requirements and specifications

| Sr.No. | Name | Notation | Unit | Value |
| --- | --- | --- | --- | --- |
| 1. | Ultimate Torque | Tmax | Nm | 3500 |
| 2. | Max. Speed of shaft | Nmax | Rpm | 6500 |
| 3. | Length of Shaft | L | Mm | 1250 |

Steel (SM45C) used for automotive drive shaft applications.  The  materialproperties of the steel (SM45C) are given in Table. The steel drive  shaftshould  satisfy three design specifications such as torque  transmission  capability,  buckling  torque capability and bending naturalfrequency.

Table 4.2: Mechanical properties of Cast iron (SM45C)

| Sr.No | Mech. Properties | Symbol | Units | Cast Iron |
| --- | --- | --- | --- | --- |
| 1. | Young’s Modulus | E | GPa | 105.0 |
| 2. | Shear Modulus | G | GPa | 36.75 |
| 3. | Poisson Ratio | v | ------ | 0.23 |
| 4. | Density | ρ | Kg/m3 | 7209 |
| 5. | Yield Strength | Sy | MPa | 130 |
| 6. | Shear Strength | Ss | MPa | 169 |

## 4.1 DesignAssumptions

- The shaft rotates at a constant speed about its longitudinalaxis.

- The shaft has a uniform, circular crosssection.

- The  shaftis  perfectlybalanced,  i.e.,  at  every  cross  section,   the   mass   center coincides with the Geometriccenter.

- All damping and nonlinear effects areexcluded.

- The	stress-strain relationship  forcompositematerial  is  linear  &elastic;  hence,  Hooke’s law is Applicable for composite materials.

- Acoustical fluid interactions are neglected, i.e., the shaft is assumed to be acting  ina vacuum.

- Since lamina is  thin  and  no  out-of-plane  loads  are  applied,  it  is  considered  as  under the planeStress.

Selection ofCross-Section:

The drive shaft can be solid circular or hollow circular. Here hollow circular cross-section waschosen because: The hollow circular shafts are strongerin  per  kg weight than solid Circular. The stress distribution in case  of  solid  shaft  is  zero  at  the center and maximum at  the  outer  surface  while  in  hollow  shaft  stress  variation  is  smaller. In solid shafts the material close to the center are not fullyutilized.

Selection ofMaterials :

Based on the advantages discussed earlier, the E-Glass/Epoxy, High Strength Carbon/Epoxy and High Modulus Carbon/Epoxy materials are  selectedfor  composite drive shaft. The Table shows the properties of the E-Glass/Epoxy, High Strength Carbon/Epoxy and High Modulus Carbon/Epoxy materials  usedfor  composite  drive shafts.

Factor ofSafety:

The designer must take into account the factor of  safetywhen  designing  a structure. Since, composites are highly orthotropic and  theirfractureswere  not  fully studied the factor of safety was taken as2.

Designcalculation:

- Inner Diameter of shaft (di) = 0.026m

- Outer Diameter of shaft (do) = 0.028m

- Length of shaft (L) = 0.335m

- Number of teeth =16

- Gear Pitch (P) =MT/2

Transmission ofTorque :

Action and reaction my friend.If a person does not turn the  pedalthen hewill stand on it and so the  maximum torque  will = (body mass  of the  rider  x g)  x the  length of the pedal lever. Remember to consider the gearing of the bikethough.

The average, fit, adult  ridercan produce  only  75  watts  or 1/10hp  when cycling at a continuous 12mph (19.3kph)." This usually  happenswith  a  pedaling  speed  of 60- 80 rpm though many rider pedals faster. When  I  cycle,  I  usually spin at between 100- 120 rpm, but I have been riding for years and have found that the  higher speed works  better for me. Typically a shaft has  acircular  cross  section.  However,  theshaft  with other cross-sections finds special application.in the design concept of a drive shaft

subjected to a fixed load. A driveshaftissupportedbygears(bearings)atboththe ends (and at  regularinterval  in  the  centrefor  longer  shafts).  The  footpedal arrangement usually placed over the shaft in between thegears.

If a device is rotating at a steady rate or  is  stationary,  the  torques appliedto it must add up to zero  --  any torque  applied  at one  point  must  be  taken off at another.So, with a bicycle crank set, the torque applied at the pedals is equal and opposite  thattaken off by the chain, neglecting  the  small  amount  lost  in  friction.  Let's  lookat  a bicycle drive train starting with the cyclist's  feet.  Torque  isconveyedfroma  pedal through the crank -- and from the left  crank,  then  also  through  the  bottom-bracket spindle -- tothe chain wheel. Generally, the cyclist's rising leg appliesa  lighttorque opposite that of the descending leg. The chain, at the  chain  wheel,  produces  a  torque equal and opposite the sum of the  torques  applied  at  the  pedals.  Let's  put  some numbers to this weassume  that  the  left  leg  is  descending  in  mid-stroke,  applying  a force of 100 pounds directly downward on the left pedal, while the rising right  leg  is applying a reverse force  of 10  pounds  to  the  right  pedal.  Crank  length  is  usually given in  millimeters,  but  we're   using  English  measurement  here,  as  it's   more   familiar   to most English-speaking readers. We'll  assume0.56  foot  (170  mm)  cranks.  The  torqueon the bottom-bracket spindle is  56  pound-feet:  the  100-pound  force  at  the  pedal, times the 0.56 foot length of the crank. The torque at the chain wheel is slightlyless,

50.4 pound feet, after we subtract the -5.6 pound-foot torque from the right pedal.

In these calculations, we neglect forces which would not  contributeto  torque: pedal force not in the direction of rotation, and the weight on the rear wheel. We also neglect friction, which reduces the drive force by a few percent. Numbers arerounded

-- close, but not exact.

The ratio of the force at the pedals to drive force at  the  road is  the  gainratio,which can be calculated more simply as  the  ratio  of road  speed  to  pedal speed,  like this:

16 teeth/10 teeth = 1.6

    - Action and reaction my friend.  If a person does not turn  the  pedal thenhe will  standon it and so the maximum torque will be“Tmax”,

Tmax = {body mass of the rider x g} x the length of the pedal lever

= 0.008*16/2 = 0.128/2 = 0.064 m

    - Module (m) = 0.008m

    - Mass Moment of Inertia (I) =MR2/2

= 4*0.0142 = 0.0039

    - Polar Moment of Inertia (J) = π (do4 –di4)/32

= π (0.0284 – 0.0264)/32 = (4.953X10-7)/32

= 1.548 X 10-8

    - Maximum Torque on bicycle is given by T = (Mass of rider x g) L Where, L = Length of pedal crank inm

‟ g = 9.81 m/sec2 (Assume mass of rider = 60 kgs) T = 60 x 9.81 x 0.335 = 197.2 Nm

    - Power(P)=2πNT/60=(2πx110x197.2)/60=2271.5watts

    - Shear Stress (τ) = Tρ/J = (197.2)(7209) /1.548 X 10-8 = 9.18 X 1013N/m4

    - Maximum Shear Stress (𝜏max) = TRo/J = (197.2)(0.014) / (1.548 x10-8)

= 17.83 X 107

    - Bending moment (M)=E/R

Where, E = Young’s modulus  I = Moment of Inertia R = Radius (Ro)

M = (105 X 0.0039)/ 0.014 = 29.25

    - ϴ= TL/GJ=(197.2)(0.335)/(36.75)(1.548 X10-8)

= 66.06/(5.68X10-7) = 1.163X109

    - Torsion is the twisting of an object due to an applied torque. It is expressed in newton metres(N-m), In sections perpendicular to the torque axis,  the  resultant  shear  stress in this section is perpendicular  to  the  radius

Where,

o T is the applied torque Nm.

- τ is the maximum shear stress at the outersurface

- r is the distance between the rotationalaxis

- ℓ is the length of the object the torque is being applied to orover.

- θ is the angle of twist inradians.

- G is the shear modulus or more commonly the modulus of rigidity(GPa),

- ro outerradius

oTorsion  (T) =  (1.548 X 10-8)(36.75)(1.163 X 109) / 0.335

Torsion (T) = 1974.9 Nm

24

Limits to torque and drive force

- Now, let's look at the limits to torque and to driveforce.

- The limit to drive  force  is  set  by  front-wheel  lifting,  which  occurs  when  the drive force is about 1/2 the weight of the cyclist andbicycle.

- Let'sassume that theweightis200pounds.Then  thedrive  force  cannot exceed 100 pounds, or the front wheel willlift.

- Let's also assume that the cyclist can push down on a pedal with a force of 200 pounds -- somewhat morethanthe  cyclist'sown  weight,  by  standing  on  a pedal and pulling up on thehandlebar.

- Then at gain ratios  below  2,  the  cyclist's  pedaling  force  is  capable   of generating over 100 pounds  of  drive  force,  and  lifting  the  front  wheel.  The cyclist must avoid pedaling as hard as possible.

- In our example with the 0.56 foot crank length and 1.11 foot wheel radius, this transition occurs almost exactly where the number of  chainwheel  teeth  and  of rear sprocket teeth is equal. So, for  example,  if  the  chain  wheel  has  28  teeth and the sprocket, 30 teeth, a hard pedal stroke can lift the frontwheel.

Stress-StrainRelationship

The drive shaft with two gears experience  twokinds  of stresses,  bending stress and shear stress. The maximum bending stress generated at the  outermost  fiber of the shaft. And on the  other  hand,  the  shear  stress  is  generated  at  the  inner  most  fiber. Also, the value of maximum bending stress is much more than the shear stress.  So, the design of the shaft will be based  onthe  maximum  bending  stress andwill be driven by  the followingformula:

Maximum bending stress Tb = (M * r) / I

Where,

M is maximum bending moment on the shaft.

r is the radius of the shaft.

I is area moment of inertia of the shaft.

  - DesignOptimization

Optimization  of  an  engineering  design  is  an  improvement  of  a  proposed design that results in the best  properties  for  minimum  cost.  Most  ofthe  methods  used for design optimization assume that the design variables are continuous. In structural optimization, almost all design variables arediscrete.

Chapter 5

Manufacturing Process

5.1. Introduction

Manufacturing involves turning raw material to finished products, to be used for various purposes.

There are a large number of processes available. These processes can be broadly classified into four

categories.

- Casting process

- Forming process

- Fabrication process

5.2. CASTING PROCESS

These processes only processes where the liquid metal is used. Casting is also the oldest known

manufacturing process. Basically, it consists of inducing the molten metal into a cavity of mould of the required form and allowing the metal to solidify. Casting is the most flexible and cheapest method

and given high strength of rigidity to the parts which are difficult to produce by other manufacturing

processes. The principle process among these sand casting where sand is used as the raw material. The

process is equally suitable for the production of a small batch as well as on a large scale. Some of the

other classified casting processes for specialized need are :-

      - Shell mould casting

      - Precision mould casting

      - Plaster mould casing

5.3. Forming process

These are solid state manufacturing processes involve minimum amount of material wastage. In

forming process metal may be heated to temperature which is slightly below. These solidify

temperature and large force is applied such the material flows and act in desired shape. The desire

shape is controlled by means of a set of tool ties and dies, which may be closed during manufacturing

These processes are normally used for large scale production rates. These are generally economical

and, in many cases, improve the mechanical properties. These are some of the metal forming

processes.

- Rolling forging

- Drop forging

- Press forging

- Upset forging

5.4. Fabrication Process

These are secondary manufacturing processes where the starting raw materials are produced by any

one of the previous manufacturing processes desired. Its assembly involves joining pieces either

temporary or permanent. So that they would be perform the necessary function. The joining can be

achieved by either or both of heat and pressure joining materials. Many of the steel structure

construction, we see are first rolled and then joined together by a fabrication process are

- Gas welding

- Electric arc welding

- Electrical resistance welding

- Thermo welding

5.5. Material Removal Process

These are also a secondary removal manufacturing process, where the additional unwanted material is

removed in the form of chips from the blank material by a hard tool so as to obtain the final desired

shape. Material removal is normally a most expensive manufacturing process. Because more energy is

consumed and also a lot of waste material is generated in this process. Still this process is widely used

because it delivers very good dimensional accuracy and good surface finished. Material removal

process are also called machining processes as shown in figure 8,9,10 and figure11. Various processes

in this category are

- Turning

- Drilling

- Shaping and planning

- Milling

- Grinding

Fig 5.1. Drilling Operations

Fig 5.2. Turning Operations

Fig 5.3. Grinding Operation    						Fig 5.4. Milling Operation

5.6. Welding

Welding is a process of joining two metal pieces by the application of heat. Welding is the least

expensive process and widely used now a days in fabrication. Welding joints different metals with the

help of a number of processes in which heat is supplied either electrically or by mean of a gas torch.

Different welding processes are used in the manufacturing of Auto mobiles bodies, structural work,

tanks, and general machine repair work. In the industries, welding is used in refineries and pipe line

fabrication. It may be called a secondary manufacturing process as shown in figure 12 and 13.

- Types of Welding Process

- GAS WELDING

- Oxy-Acetylene

- Air-Acetylene

- Oxy-Hydrogen

- Resistance Welding

- Butt Welding

- Spot Welding

- Seam Welding

- Projection Welding

- Arc Welding

- Carbon Arc Welding

- Metal Arc Welding

- Plasma Arc Welding

- Gas Metal Arc Welding

- Gas Tungsten Arc Welding

- Newer Welding

- Electron Beam Welding

- Laser Beam Welding

Fig 5.5. Arc Welding

- Advantages of Shaft Driven bicycle are :

- Drive system is less likely tobecome jammed,acommonproblemwithchain-driven bicycles.

- The rider cannot become dirtied fromchain  greaseor  injured  by  the  chain  from  "Chain bite", which occurs when clothing or even a body partcatchesbetweenthechainanda sprocket.

- Lower maintenance  than  a chain  system  when the  drive  shaft  is  enclosed  in atube.

- Dynamic Bicycles claims that a drive shaft bicycle can deliver 94% efficiency, whereas a chain-driven bicycle can deliver anywhere from 75-97% efficiency based oncondition.

- Greater clearance: with the absence of a derailleur orother  low-hanging  machinery,  the  bicycle has nearly twice the groundclearance.

- Another advantage that may be noticed about the chainless bicycle is that  itprotects  your clothes better. Your footwear and your pants do not get accidentally damaged, and  youdo not  have the same amount of cleaning to do. However, those who are not socrazyaboutthis alternative type of bicycle  say  that  it  is  enough  to  choose  a  bicycle  with  encased  chain,  or with chain guards, and the problem issolved.

Limitations of this cycle :

- A drive shaft system weighs more than a chain system, usually 1-2 pounds heavier.

- Many of the advantages claimed by drive shaft's proponents can be  achievedon  a  chain driven bicycle, such as covering the chain and gears with a metal or plasticcover.

- Use  of  lightweight  derailleur  gears  with  a  high  number  of  ratios  is  impossible,  although hub.

- Gears can beused.

- Wheel removal can be complicated in some designs (as it  isfor  some  chain-driven bicycles with hubgears).

TROUBLESHOOTING

  - When abnormal vibrations or noises  are  detected  in  the  driveshaft  area,  this  chart  can be used to help diagnose possible causes. Remember that other components such as wheels, tires, rear axle and suspension can also produce similarconditions.

Table 5.1 :

| Problem | Caused by | What to do |
| --- | --- | --- |
| As bicycle is accelerated from Stop | torque is required | Apply more torque at starting |
| when gears are not shifting | Rusting | Clean with fluids |
| Vibration at speed | High speed | Maintain low speed |
| Noise at low speed | Universal joint | Apply grease |
| Gears pitch circle is not coincide | Vibrations | Adjust the position of gears |
| Gear backlash | Noise, Overloading, Overheating | Follow design characteristics |

CONCLUSION

  - Firstly the project were unable to  be completed with the drive shaft due  to  various problems around circumference of the bicycle ,later on this wasrealizedto runsuccessfully with two bevel gears at both end of thedrive shaft

  - The presented work was aimed to reduce the wastage of  human  power  (energy)  on bicycle  riding  or  any  machine,  which  employs  drive  shafts;  in  general  it   is   achieved by using light weight drive shaft with bevel gears  on both sides  designed  on replacing  chain transmission.

  - The  presented  work  alsodeals  with  design  optimization  i.e.  converting  rotarymotion in linear motion with aid of two bevelgears.

  - Instead of chain drive one piece drive shaft for rear wheel drive bicycle  havebeen  optimally designed and manufactured for easily powertransmission.

  - The drive shaft with the objective of  minimization  of  weight  of  shaft  which  was  subjected to the constraints such as torque transmission ,  torsion  buckling  capacity  , stress, strain ,etc

  - The torque transmission capacity of the bicycle drive shaft has  been  calculated  by neglecting and  considering  the  effect  of  centrifugal  forces  and  it  has  been  observed that centrifugal force will reduce the torque transmission capacity of theshaft.

  - The stress distribution  and  the  maximum  deformation  in  the  drive  shaft  are  the functions of the stacking of material. The optimum  stackingof  material  layers  can  be used as the effective tool to reduce weight and stress acting on the driveshaft.

  - The design of drive shaft is critical as it is subjected to combined loads. Thedesigner

has two options for designing  the  drive  shaft  whether  to  select  solid  or  hollow  shaft.  The solid shaft gives a maximum value of torque transmission but at same time due  to increase in weight of shaft,  For  a  given  weight,  the  hollow  shaft  is  stronger because it has a bigger diameter due to less weight &less bending moment.

The results obtained from this work is  an  useful  approximation  to  help  in  the  earlier  stages  of the development, saving development time and  helping  in  the  decision  making  process  to optimize adesign.

REFERENCES

- Rastogi, N. (2004). Design of composite drive shafts for automotive applications. Visteon Corporation, SAE technical paperseries.

- Design and Analysis of a Propeller Shaft of a Toyota Qualisby “SyedHasan”.

- A.M.Ummuhaani and Dr.P.Sadagopan“Design, Fabrication and Stress Analysis of a Composite Propeller Shaft,2011-28-0013.

- Anup A. Bijagare, P.G. MeharandV.N. Mujbaile“Design  Optimization  &Analysis of Drive Shaft”, Vol. 2 (6), 2012,210-215.

- Rangaswamy, T.; Vijayrangan, S. (2005). Optimal sizing and stacking sequence of composite drive shafts. Materials science, Vol. 11 No 2.India.

- Rastogi, N. (2004). Design of composite drive shafts for automotive applications. Visteon Corporation, SAE technical paperseries.

- R. P. Kumar Rompicharla1, Dr. K. Rambabu2 Sep-Oct. 2012 Design and Optimization of Drive Shaft with Composite Materials International Journal  of Modern Engineering Research (IJMER) www.ijmer.comVol.2, Issue.5, pp-3422- 3428 ISSN:2249-6645.

WEBSITE REFERENCES

- Dynamicbicycles.com

- www.engineeringtoolbox.com

- Makeitform.com

- Convertunits.com

- Webbicycle.netpaths.net/technology.php